1D Axial Finite Element Analysis of Hollow Human Femur Under Compressive Loading

# Abstract

This study presents a one–dimensional finite element analysis (FEA) of a simplified hollow human femur subjected to axial compressive loading. The femur shaft is modeled as a hollow cylindrical cortical bone segment and analyzed using a custom Python-based FEM solver. The numerical results are validated through ANSYS Mechanical simulation under identical geometric, material, and loading conditions. The study evaluates axial deformation, stress distribution, and structural safety based on cortical bone yield and ultimate strength limits. The results demonstrate consistency between analytical FEM implementation and commercial simulation outputs, confirming the validity of the simplified biomechanical model.

# 1. Introduction

The femur is the longest and strongest bone in the human body, designed to withstand substantial mechanical loads during activities such as walking, running, and jumping. Understanding the stress–strain behavior of the femur under compressive loading is essential in biomechanics, orthopedic implant design, and injury mechanics. Finite Element Analysis (FEA) provides a powerful numerical framework for analyzing biomechanical structures under realistic loading conditions.

This project focuses on the axial compressive response of a simplified hollow femur shaft using a 1D bar element formulation. The primary objective is to compute axial deformation and stress distribution and to validate the numerical results using ANSYS Mechanical simulation.

# 2. Objectives

The main objectives of this project are:

- To model the femur as a hollow cylindrical bar

- To perform axial finite element analysis using ANSYS

- To develop a custom Python code for 1D FEA

- To compare analytical, numerical, and software-based solutions

- To evaluate axial stress and displacement distribution3

# 3. Assumptions

The following assumptions are made:

- The femur is modeled as a hollow cylindrical bar

- Material behavior is linear elastic

- Cross-section is uniform along the length

- Only axial loading is considered

- Bending and torsion effects are neglected

- Bone is considered isotropic for simplification

# 4. Theoretical Background

The hollow cylindrical bar is subjected to axial load.

Axial stress (σ)=P/A

Where:

P = Axial Load

A = Cross-sectional area

For hollow selection:   A=π4D2-d2

Where:

D = Outer Diameter

D = Inner Diameter

Axial Deformation (δ)=PLAE

Where:

L = Length

E = Young’s Modulus

In Finite Element Method: [K]{u} = {F}

Where:

K = Global Stiffness Matrix

u = Nodal Displacement Vector

F = Force Vector

# 5. Geometry and Cross‑Section

The femur shaft is modeled as a hollow cylindrical structure representing cortical bone. The geometric parameters used in the analysis are:

| Parameter | Value |
| --- | --- |
| Length | 0.48 m |
| Outer Diameter | 25.1 mm |
| Inner Diameter | 11.3 mm |

The cross-sectional area is calculated using the formula for a hollow circular section.

# 6. Material Properties

The femur cortical bone is assumed to be linear elastic, homogeneous, and isotropic.

| Property | Value |
| --- | --- |
| Young’s Modulus | 16 GPa |
| Yield Strength | 90 MPa |
| Ultimate Strength Poisson Ratio | 160 MPa 0.31 |

# 7. Finite Element Formulation

The femur shaft is discretized into ten linear axial bar elements. The element stiffness matrix is derived from the standard 1D bar formulation. Global stiffness assembly is performed, followed by application of boundary conditions and solution of nodal displacements using numerical linear algebra techniques.

# 8. Loading and Boundary Conditions

A compressive axial load of 50,000 N is applied at the distal end of the femur, while the proximal end is fully constrained. This loading condition represents a high physiological load scenario.

# 9. Results and Discussion

The Python FEM model computes axial displacement distribution along the femur shaft and evaluates the corresponding stress values. The maximum compressive stress is compared with cortical bone yield and ultimate strength limits to determine structural safety.

The deformation results are found to be within micrometer range, indicating high axial stiffness of cortical bone. The stress distribution remains uniform due to the constant cross‑section assumption. The computed stress is below the yield strength, indicating elastic structural behavior.

# 10. ANSYS Validation

A corresponding finite element model is developed in ANSYS Mechanical using identical geometry, material properties, and loading conditions. The total deformation results obtained from ANSYS show strong agreement with the Python FEM solution, validating the correctness of the numerical implementation.

# 11. Conclusion

This study demonstrates the effectiveness of simplified 1D finite element modeling in predicting axial mechanical response of the femur shaft. The agreement between Python and ANSYS results confirms the validity of the implemented FEM formulation. The model provides a foundational framework for further biomechanical studies involving complex loading conditions and realistic bone geometry.

# 12. Future Scope

Future extensions of this work may include nonlinear material modeling, 3D femur geometry analysis, trabecular‑cortical interaction, fracture prediction, fatigue analysis, and patient‑specific biomechanical simulation.

By- Jiveeka Negi